| CUSTOMER | TI |
| --- | --- |
| CARD NAME | 1642-EVM-REV-A |
| PART NUMBER | TS13730AC0 |
| DATE | 03/02/17 |
| DESIGNED AT | Tessolve Semiconductor Pvt. Ltd. |

| GERBER FORMAT | RS274X , ENGLISH 2,5 LEADING |
| --- | --- |
| DRILL FORMAT | EXCELLON ENGLISH 2,5 TRAILING |
| PACKAGE NAME | TS13730AC0_1642-EVM-REV-A_FAB_20170302.zip |

LIST OF FILES INSIDE PACKAGE:

ELECTRICAL LAYERS :

L01.art		- Layer 1 (Top)

L02P.art		- Layer 2 (Ground)

L03.art		- Layer 3 (Gnd/signal)

L04.art		- Layer 4 (Gnd/signal)

L05P.art		- Layer 5 (Ground)

L06.art		- Layer 6 (Bottom)

NON ELECTRICAL LAYERS :

SMT.art		- Solder Mask Top Side

SMB.art		- Solder Mask Bottom Side

SST.art		- Silkscreen Top Side

SSB.art		- Silkscreen Bottom Side

OUTLINE.art	- Board Outline

FAB-1.art		- Fabrication Drawing 1

FAB-2.art		- Fabrication Drawing 2

DRILL LAYERS :

L1-L2.drl		- Blind via (Blind via from L1 to L2)

PTH.drl		- Plated through holes drill file

NPTH.drl		- Non Plated through holes drill file

PTH_ROUTE.rou 	- Plated slot/cutout Route file

OTHER FILES :

IPCD356.ipc					- IPC-D-356 Netlist

Layers.pdf						- PDF Plots of all layers

Fab.pdf						- PDF of Fabrication Drawing

AR1642-EVM-011-Stackup.pdf		- Stack-up PDF

Via_Filling_Requirements.docx  		- Via filling document

Antenna_etching_Requirements.docx	- Antenna etching document

Intentional Short.txt		   		- Intentional Net short report

Readme_Fab.doc					- This file

BOARD SPECIFICATIONS :

Board Size				 : 3.3”  X 2.565”

No. of Layers			 :	6

Board Thickness			 : 0.05621”

Board Finish			 : Immersion Silver

Min. Trace width and Spacing  : 0.004” / 0.004”

Min. Hole Size			 : 0.0059”(blind via) and 0.0122”(through hole)

Impedance Control		 : YES