|  |  |  |  |  |  |  | QR-ME-15(A) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Engineering Confirmation Note Layer stackup & impedance |  |  |  |  |  |  |  |
| Subject : questions about the stack and the impedance value |  |  |  |  |  |  |  |  |  |
| From ：Fast-print Circuit Tech Co., Ltd |  |  |  |  |  |  |  |  |  |
| Writer: |  |  |  |  |  |  |  |  |  |
| Check: | Steven Cao |  |  |  |  |  |  |  |  |
| DATE: | 2012-11-6 14:30 |  |  |  |  |  |  |  |  |
| Customer Part NO： |  |  |  |  |  |  |  |  |  |
| Customer Requirements |  |  |  |  |  |  |  |  |  |
| Layer | Line Width/Space(mil) |  | Control Impedance（ohm） |  |  |  |  |  |  |
| L1 | 6.2 mil |  | 50+/-10% |  |  |  |  |  |  |
| Our Suggesting： |  |  |  |  |  |  |  |  |  |
| L1 |  |  |  |  | 0.5oz +Plating |  |  |  |  |
|  | 2116 4.4345 mil |  |  |  |  |  |  |  |  |
| P2 |  |  |  |  | 1oz |  |  |  |  |
|  | Core 48.441 mil |  |  |  |  |  |  |  |  |
| P3 |  |  |  |  | 1oz |  |  |  |  |
|  | 2116 4.4345 mil |  |  |  |  |  |  |  |  |
| L4 |  |  |  |  | 0.5oz +Plating |  |  |  |  |
| Overlap THK: |  |  | 1.56 mm |  | Meterial : | FR4 IT-180A |  |  |  |
| Finish THK: |  |  | 1.6 +/-0.16 mm |  |  |  |  |  |  |
| Calculate Impedance： |  |  |  |  |  |  |  |  |  |
| Layer | Adjust LW/LS(mil) |  | Calculate Value(Ω) |  |  | H1(mil) | Er1 | H2(mil) | Er2 |
| L1 | 6.2 mil |  | 50.4 |  |  | 4.43 | 4.3 |  |  |
| L1 | Ground is: | L2 |  |  |  |  |  |  |  |
| Customer Reply: |  | Accept: | Disapprove: |  |  |  |  |  |  |
| Signature： |  | Date: |  |  |  |  |  |  |  |
| Remark : |  |  |  |  |  |  |  |  |  |